EARLY RElaPSE RISK DETECTION USING WEARABLE PHYSIOLOGICAL SIGNALS IN AUD

FINAL REPORT

ADARSH KONDERU

C5051863

MSC ARTIFICAL INTELLIGENCE

MODULE LEADER:

GREGOR MILLIGAN

AI DEVELOPMENT PROJECT

Introduction

Nowadays, wearable devices are helping us to monitor physiological signals in real time. These signals are providing useful information about person’s physical and emotional state. One of the important applications is stress detection.

Stress is identified as one of the major factors that can leads to relapse in Alcohol Use Disorder (AUD). Being able to find stress early can help in identifying individuals who are at the risk of relapse.

This project mainly focuses on using wearable physiological signals to detect stress using machine learning techniques.

Problem Statement

The main problem addressed in this project is that limited options and methods are available for continuously monitoring stress levels.

Most of the existing methods rely on self-reporting, which may not always be accurate. Therefore, we need an automated system that can detect stress based on physiological data.

The aim of this project is to identify whether a person is in a baseline (normal) state or a stress state using wearable sensor data.

Dataset Description

In this project I have used WESAD dataset, which contains physiological data collected from wearable devices.

The following signals were used:

* Electrodermal Activity (EDA)

* Blood Volume Pulse (BVP)

* Skin Temperature (TEMP)

The dataset contains 15 participants samples overall data of both wrist and heart related sensors

Each data sample is labelled as:

* Baseline (1)

* Stress (2)

Methodology

The overall process followed in this project includes data preprocessing, feature extraction, model training, and evaluation.

Signal Processing

The raw signals were first extracted from the dataset. These signals were continuous time-series data.

Windowing

The signals were divided into smaller segments using a window size of 200 samples, which corresponds to approximately 50 seconds. This helps in analysing the data in smaller chunks.

Feature Extraction

From each window, simple statistical features were extracted:

* Mean

* Standard Deviation

These features help in summarising the behaviour of the signals.

Model Selection

A Random Forest classifier was used for classification. This model was chosen because it performs well on structured data and can handle complex patterns.

Train-Test Split

The dataset was split into:

* 80% training data

* 20% testing data

This ensures that the model is evaluated on unseen data.

The implementation was carried out using Python in Google Colab, using libraries such as NumPy, Pandas, Matplotlib, Seaborn, and Scikit-learn.

Justification

Random Forest was selected for this project because it performs well with structured data and can handle nonlinear relationships between features. It is also good for noise data, which is useful when working with physiological signals.

Simple statistical features such as mean and standard deviation were used because they are easy to compute and provide a good summary of the signal behaviour. This approach keeps the model simple while still achieving good performance.

Results

The model has  achieved an accuracy of approximately 87% on the test dataset.

From the confusion matrix:

* 97 samples were correctly classified

* 14 samples were incorrectly classified

The model showed good performance, especially in detecting stress cases, which is important for this application.

Discussion

The results shows that physiological signals can be effectively used to detect stress.

However, there were some misclassifications. This is mainly due to:

* Overlap between stress and baseline signals

* Variability between different individuals

* Use of simple features

Even though the accuracy slightly decreased when using more participants, the results became more realistic and reliable.

Conclusion

This project concludes that stress detection using wearable physiological signals is possible using machine learning techniques.

The Random Forest model was able to classify stress and baseline states with good accuracy. Since stress is a major factor in relapse, this approach can be useful for early relapse risk detection.

Limitations and Future Work

Limitations

* The dataset does not include actual relapse data, so stress is used as an indirect indicator

* Only basic statistical features were used

* There can be overlap between stress and baseline signals

Future Work

* Use more advanced features from the signals

* Use deep learning models such as CNN or LSTM

* Use large and more relevant datasets

* Develop a real-time monitoring system using wearable devices